--AXI-Lite wrapper

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

entity myip_12_v1_0_S00_AXI is

generic (

C_S_AXI_DATA_WIDTH : integer := 32;

C_S_AXI_ADDR_WIDTH : integer := 5

);

port (

S_AXI_ACLK    : in  std_logic;

S_AXI_ARESETN : in  std_logic;

S_AXI_AWADDR  : in  std_logic_vector(C_S_AXI_ADDR_WIDTH-1 downto 0);

S_AXI_AWPROT  : in  std_logic_vector(2 downto 0);

S_AXI_AWVALID : in  std_logic;

S_AXI_AWREADY : out std_logic;

S_AXI_WDATA   : in  std_logic_vector(C_S_AXI_DATA_WIDTH-1 downto 0);

S_AXI_WSTRB   : in  std_logic_vector((C_S_AXI_DATA_WIDTH/8)-1 downto 0);

S_AXI_WVALID  : in  std_logic;

S_AXI_WREADY  : out std_logic;

S_AXI_BRESP   : out std_logic_vector(1 downto 0);

S_AXI_BVALID  : out std_logic;

S_AXI_BREADY  : in  std_logic;

S_AXI_ARADDR  : in  std_logic_vector(C_S_AXI_ADDR_WIDTH-1 downto 0);

S_AXI_ARPROT  : in  std_logic_vector(2 downto 0);

S_AXI_ARVALID : in  std_logic;

S_AXI_ARREADY : out std_logic;

S_AXI_RDATA   : out std_logic_vector(C_S_AXI_DATA_WIDTH-1 downto 0);

S_AXI_RRESP   : out std_logic_vector(1 downto 0);

S_AXI_RVALID  : out std_logic;

S_AXI_RREADY  : in  std_logic

);

end myip_12_v1_0_S00_AXI;

architecture arch_imp of myip_12_v1_0_S00_AXI is

signal axi_awaddr  : std_logic_vector(C_S_AXI_ADDR_WIDTH-1 downto 0);

signal axi_awready : std_logic;

signal axi_wready  : std_logic;

signal axi_bresp   : std_logic_vector(1 downto 0);

signal axi_bvalid  : std_logic;

signal axi_araddr  : std_logic_vector(C_S_AXI_ADDR_WIDTH-1 downto 0);

signal axi_arready : std_logic;

signal axi_rdata   : std_logic_vector(C_S_AXI_DATA_WIDTH-1 downto 0);

signal axi_rresp   : std_logic_vector(1 downto 0);

signal axi_rvalid  : std_logic;

signal aw_en       : std_logic;

constant ADDR_LSB          : integer := (C_S_AXI_DATA_WIDTH/32) + 1; -- 3

constant OPT_MEM_ADDR_BITS : integer := 2;  -- loc_addr 0..4

-- 4 plaintext registers + 1 control register

signal slv_reg0 : std_logic_vector(C_S_AXI_DATA_WIDTH-1 downto 0);

signal slv_reg1 : std_logic_vector(C_S_AXI_DATA_WIDTH-1 downto 0);

signal slv_reg2 : std_logic_vector(C_S_AXI_DATA_WIDTH-1 downto 0);

signal slv_reg3 : std_logic_vector(C_S_AXI_DATA_WIDTH-1 downto 0);

signal ctrl_reg : std_logic_vector(C_S_AXI_DATA_WIDTH-1 downto 0);  -- written by SW

signal slv_reg_rden : std_logic;

signal slv_reg_wren : std_logic;

signal reg_data_out : std_logic_vector(C_S_AXI_DATA_WIDTH-1 downto 0);

signal byte_index   : integer;

-- AES core signals

signal aes_key           : std_logic_vector(127 downto 0);

signal aes_plain         : std_logic_vector(127 downto 0);

signal aes_cipher        : std_logic_vector(127 downto 0);

signal aes_load          : std_logic;

signal aes_ready         : std_logic;

signal aes_empty         : std_logic;

signal aes_reset         : std_logic;

signal status_ready_latched : std_logic;

component aes128key is

port(

reset  : in  std_logic;

clock  : in  std_logic;

empty  : out std_logic;

load   : in  std_logic;

key    : in  std_logic_vector(127 downto 0);

plain  : in  std_logic_vector(127 downto 0);

ready  : out std_logic;

cipher : out std_logic_vector(127 downto 0)

);

end component;

begin

S_AXI_AWREADY <= axi_awready;

S_AXI_WREADY  <= axi_wready;

S_AXI_BRESP   <= axi_bresp;

S_AXI_BVALID  <= axi_bvalid;

S_AXI_ARREADY <= axi_arready;

S_AXI_RDATA   <= axi_rdata;

S_AXI_RRESP   <= axi_rresp;

S_AXI_RVALID  <= axi_rvalid;

--------------------------------------------------------------------

-- Write address ready

--------------------------------------------------------------------

process (S_AXI_ACLK)

begin

if rising_edge(S_AXI_ACLK) then

if S_AXI_ARESETN = '0' then

axi_awready <= '0';

aw_en       <= '1';

else

if (axi_awready = '0' and S_AXI_AWVALID = '1' and

S_AXI_WVALID = '1' and aw_en = '1') then

axi_awready <= '1';

aw_en       <= '0';

elsif (S_AXI_BREADY = '1' and axi_bvalid = '1') then

aw_en       <= '1';

axi_awready <= '0';

else

axi_awready <= '0';

end if;

end if;

end if;

end process;

-- Latch AWADDR

process (S_AXI_ACLK)

begin

if rising_edge(S_AXI_ACLK) then

if S_AXI_ARESETN = '0' then

axi_awaddr <= (others => '0');

else

if (axi_awready = '0' and S_AXI_AWVALID = '1' and

S_AXI_WVALID = '1' and aw_en = '1') then

axi_awaddr <= S_AXI_AWADDR;

end if;

end if;

end if;

end process;

--------------------------------------------------------------------

-- WREADY

--------------------------------------------------------------------

process (S_AXI_ACLK)

begin

if rising_edge(S_AXI_ACLK) then

if S_AXI_ARESETN = '0' then

axi_wready <= '0';

else

if (axi_wready = '0' and S_AXI_WVALID = '1' and

S_AXI_AWVALID = '1' and aw_en = '1') then

axi_wready <= '1';

else

axi_wready <= '0';

end if;

end if;

end if;

end process;

--------------------------------------------------------------------

-- Register writes

--------------------------------------------------------------------

slv_reg_wren <= axi_wready and S_AXI_WVALID and axi_awready and S_AXI_AWVALID;

process (S_AXI_ACLK)

variable loc_addr : std_logic_vector(OPT_MEM_ADDR_BITS downto 0);

begin

if rising_edge(S_AXI_ACLK) then

if S_AXI_ARESETN = '0' then

slv_reg0 <= (others => '0');

slv_reg1 <= (others => '0');

slv_reg2 <= (others => '0');

slv_reg3 <= (others => '0');

ctrl_reg <= (others => '0');

else

loc_addr := axi_awaddr(ADDR_LSB + OPT_MEM_ADDR_BITS downto ADDR_LSB);

if slv_reg_wren = '1' then

case loc_addr is

when b"000" =>

for byte_index in 0 to (C_S_AXI_DATA_WIDTH/8-1) loop

if S_AXI_WSTRB(byte_index) = '1' then

slv_reg0(byte_index*8+7 downto byte_index*8) <=

S_AXI_WDATA(byte_index*8+7 downto byte_index*8);

end if;

end loop;

when b"001" =>

for byte_index in 0 to (C_S_AXI_DATA_WIDTH/8-1) loop

if S_AXI_WSTRB(byte_index) = '1' then

slv_reg1(byte_index*8+7 downto byte_index*8) <=

S_AXI_WDATA(byte_index*8+7 downto byte_index*8);

end if;

end loop;

when b"010" =>

for byte_index in 0 to (C_S_AXI_DATA_WIDTH/8-1) loop

if S_AXI_WSTRB(byte_index) = '1' then

slv_reg2(byte_index*8+7 downto byte_index*8) <=

S_AXI_WDATA(byte_index*8+7 downto byte_index*8);

end if;

end loop;

when b"011" =>

for byte_index in 0 to (C_S_AXI_DATA_WIDTH/8-1) loop

if S_AXI_WSTRB(byte_index) = '1' then

slv_reg3(byte_index*8+7 downto byte_index*8) <=

S_AXI_WDATA(byte_index*8+7 downto byte_index*8);

end if;

end loop;

when b"100" =>

for byte_index in 0 to (C_S_AXI_DATA_WIDTH/8-1) loop

if S_AXI_WSTRB(byte_index) = '1' then

ctrl_reg(byte_index*8+7 downto byte_index*8) <=

S_AXI_WDATA(byte_index*8+7 downto byte_index*8);

end if;

end loop;

when others =>

null;

end case;

end if;

end if;

end if;

end process;

--------------------------------------------------------------------

-- Write response

--------------------------------------------------------------------

process (S_AXI_ACLK)

begin

if rising_edge(S_AXI_ACLK) then

if S_AXI_ARESETN = '0' then

axi_bvalid <= '0';

axi_bresp  <= "00";

else

if (axi_awready = '1' and S_AXI_AWVALID = '1' and

axi_wready = '1' and S_AXI_WVALID = '1' and

axi_bvalid = '0') then

axi_bvalid <= '1';

axi_bresp  <= "00";

elsif (S_AXI_BREADY = '1' and axi_bvalid = '1') then

axi_bvalid <= '0';

end if;

end if;

end if;

end process;

--------------------------------------------------------------------

-- Read address ready

--------------------------------------------------------------------

process (S_AXI_ACLK)

begin

if rising_edge(S_AXI_ACLK) then

if S_AXI_ARESETN = '0' then

axi_arready <= '0';

axi_araddr  <= (others => '0');

else

if (axi_arready = '0' and S_AXI_ARVALID = '1') then

axi_arready <= '1';

axi_araddr  <= S_AXI_ARADDR;

else

axi_arready <= '0';

end if;

end if;

end if;

end process;

--------------------------------------------------------------------

-- Read data valid

--------------------------------------------------------------------

process (S_AXI_ACLK)

begin

if rising_edge(S_AXI_ACLK) then

if S_AXI_ARESETN = '0' then

axi_rvalid <= '0';

axi_rresp  <= "00";

else

if (axi_arready = '1' and S_AXI_ARVALID = '1' and

axi_rvalid = '0') then

axi_rvalid <= '1';

axi_rresp  <= "00";

elsif (axi_rvalid = '1' and S_AXI_RREADY = '1') then

axi_rvalid <= '0';

end if;

end if;

end if;

end process;

--------------------------------------------------------------------

-- Read mux

--------------------------------------------------------------------

slv_reg_rden <= axi_arready and S_AXI_ARVALID and (not axi_rvalid);

process (aes_cipher, ctrl_reg, status_ready_latched, axi_araddr, slv_reg_rden)

variable loc_addr : std_logic_vector(OPT_MEM_ADDR_BITS downto 0);

variable tmp_ctl  : std_logic_vector(C_S_AXI_DATA_WIDTH-1 downto 0);

begin

loc_addr := axi_araddr(ADDR_LSB + OPT_MEM_ADDR_BITS downto ADDR_LSB);

case loc_addr is

when b"000" => reg_data_out <= aes_cipher(127 downto 96);

when b"001" => reg_data_out <= aes_cipher(95  downto 64);

when b"010" => reg_data_out <= aes_cipher(63  downto 32);

when b"011" => reg_data_out <= aes_cipher(31  downto 0);

when b"100" =>

tmp_ctl      := ctrl_reg;

tmp_ctl(0)   := ctrl_reg(0) or status_ready_latched; -- status bit

reg_data_out <= tmp_ctl;

when others =>

reg_data_out <= (others => '0');

end case;

end process;

process (S_AXI_ACLK)

begin

if rising_edge(S_AXI_ACLK) then

if S_AXI_ARESETN = '0' then

axi_rdata <= (others => '0');

else

if slv_reg_rden = '1' then

axi_rdata <= reg_data_out;

end if;

end if;

end if;

end process;

--------------------------------------------------------------------

-- AES core connection and control

--------------------------------------------------------------------

aes_plain <= slv_reg0 & slv_reg1 & slv_reg2 & slv_reg3;

aes_key   <= x"000102030405060708090A0B0C0D0E0F";          -- key fixed inside aes128key

aes_reset <= not S_AXI_ARESETN;

aes_dut : aes128key

port map (

reset  => aes_reset,

clock  => S_AXI_ACLK,

empty  => aes_empty,

load   => aes_load,

key    => aes_key,

plain  => aes_plain,

ready  => aes_ready,

cipher => aes_cipher

);

-- generate one-cycle load pulse from writes to ctrl_reg bit0

load_pulse_proc : process (S_AXI_ACLK)

variable loc_addr : std_logic_vector(OPT_MEM_ADDR_BITS downto 0);

begin

if rising_edge(S_AXI_ACLK) then

if S_AXI_ARESETN = '0' then

aes_load <= '0';

else

loc_addr := axi_awaddr(ADDR_LSB + OPT_MEM_ADDR_BITS downto ADDR_LSB);

if slv_reg_wren = '1' and loc_addr = b"100" and S_AXI_WDATA(0) = '1' then

aes_load <= '1';

elsif aes_load = '1' then

aes_load <= '0';

end if;

end if;

end if;

end process;

-- latch ready separately (no drive on ctrl_reg)

ready_latch_proc : process (S_AXI_ACLK)

begin

if rising_edge(S_AXI_ACLK) then

if S_AXI_ARESETN = '0' then

status_ready_latched <= '0';

else

if aes_ready = '1' then

status_ready_latched <= '1';

end if;

end if;

end if;

end process;

end arch_imp;