--Implementation based upon http://nvlpubs.nist.gov/nistpubs/FIPS/NIST.FIPS.197.pdf

--MIT License

-- Modified for ZYBO Z7 AXI-Lite integration and benchmarking.

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use ieee.std_logic_unsigned.all;

entity aes128key is

Port(

reset : in  STD_LOGIC;

clock : in  STD_LOGIC;

--input side signals

empty : out STD_LOGIC;

load : in  STD_LOGIC;

key : in  STD_LOGIC_VECTOR (127 downto 0);

plain : in  STD_LOGIC_VECTOR (127 downto 0);

--output side signals

ready : out  STD_LOGIC;

cipher : out STD_LOGIC_VECTOR(127 downto 0));

end aes128key;

architecture Behavioral of aes128key is

--state machine signals

signal roundCounter : std_Logic_vector(5 downto 0);

--key expansion signals

signal temp, rotW, subRotW : std_logic_vector(31 downto 0);

type wT is array(0 to 3) of std_logic_vector(31 downto 0);

signal w : wT;

type RconT is array(0 to 11) of std_logic_vector(7 downto 0);

constant Rcon : RconT := (x"01",x"02",x"04",x"08",x"10",x"20",x"40",x"80",x"1b",x"36",

x"aa",x"bb"); --last two constants needed for simulation to run properly, can be deleted before synthesis to cost less hardware

--encryption signals

signal expansionKey : std_logic_vector(127 downto 0);

signal add, sub, shift, mix, mixOut : std_logic_vector(127 downto 0);

type stateRowT is array (0 to 3) of std_logic_vector(7 downto 0);

type stateT is array (0 to 3) of stateRowT;

signal shiftInState, shiftOutState: stateT;

begin

------------------------------

--CONTROLLING STATE MACHINE---

------------------------------

fsm: process(clock)

begin

if rising_edge(clock) then

if reset = '1' then

roundCounter <= x"a" & "01";

else

if roundCounter = (x"a" & "01") then

if load = '1' then

roundCounter <= "000000";

end if;

elsif roundCounter /= (x"a" & "01") then

roundCounter <= roundCounter + '1';

end if;

end if;

end if;

end process;

ready <= '1' when roundCounter = x"a" & "00" else '0';

empty <= '1' when roundCounter = x"a" & "01" else '0';

--------------------

--KEY EXPANSION-----

--------------------

keyExpansionPipe: process(clock)

begin

if rising_edge(clock) then

if load = '1' then

w(0) <= key(127 downto 96);

w(1) <= key(95 downto 64);

w(2) <= key(63 downto 32);

w(3) <= key(31 downto 0);

elsif roundCounter < (x"a" & "00") then

w <= w(1 to 3) & (w(0) xor temp);

end if;

end if;

end process;

--begin keyExpansionPipe asynchron circuitry

rotW <=	w(3)(23 downto 0) & w(3)(31 downto 24);

subBytesFor: for i in 0 to 3 generate

type subTK is array(0 to 255) of std_logic_vector(7 downto 0);

constant subKey : subTK :=(

x"63", x"7c", x"77", x"7b", x"f2", x"6b", x"6f", x"c5", x"30", x"01", x"67", x"2b", x"fe", x"d7", x"ab", x"76",

x"ca", x"82", x"c9", x"7d", x"fa", x"59", x"47", x"f0", x"ad", x"d4", x"a2", x"af", x"9c", x"a4", x"72", x"c0",

x"b7", x"fd", x"93", x"26", x"36", x"3f", x"f7", x"cc", x"34", x"a5", x"e5", x"f1", x"71", x"d8", x"31", x"15",

x"04", x"c7", x"23", x"c3", x"18", x"96", x"05", x"9a", x"07", x"12", x"80", x"e2", x"eb", x"27", x"b2", x"75",

x"09", x"83", x"2c", x"1a", x"1b", x"6e", x"5a", x"a0", x"52", x"3b", x"d6", x"b3", x"29", x"e3", x"2f", x"84",

x"53", x"d1", x"00", x"ed", x"20", x"fc", x"b1", x"5b", x"6a", x"cb", x"be", x"39", x"4a", x"4c", x"58", x"cf",

x"d0", x"ef", x"aa", x"fb", x"43", x"4d", x"33", x"85", x"45", x"f9", x"02", x"7f", x"50", x"3c", x"9f", x"a8",

x"51", x"a3", x"40", x"8f", x"92", x"9d", x"38", x"f5", x"bc", x"b6", x"da", x"21", x"10", x"ff", x"f3", x"d2",

x"cd", x"0c", x"13", x"ec", x"5f", x"97", x"44", x"17", x"c4", x"a7", x"7e", x"3d", x"64", x"5d", x"19", x"73",

x"60", x"81", x"4f", x"dc", x"22", x"2a", x"90", x"88", x"46", x"ee", x"b8", x"14", x"de", x"5e", x"0b", x"db",

x"e0", x"32", x"3a", x"0a", x"49", x"06", x"24", x"5c", x"c2", x"d3", x"ac", x"62", x"91", x"95", x"e4", x"79",

x"e7", x"c8", x"37", x"6d", x"8d", x"d5", x"4e", x"a9", x"6c", x"56", x"f4", x"ea", x"65", x"7a", x"ae", x"08",

x"ba", x"78", x"25", x"2e", x"1c", x"a6", x"b4", x"c6", x"e8", x"dd", x"74", x"1f", x"4b", x"bd", x"8b", x"8a",

x"70", x"3e", x"b5", x"66", x"48", x"03", x"f6", x"0e", x"61", x"35", x"57", x"b9", x"86", x"c1", x"1d", x"9e",

x"e1", x"f8", x"98", x"11", x"69", x"d9", x"8e", x"94", x"9b", x"1e", x"87", x"e9", x"ce", x"55", x"28", x"df",

x"8c", x"a1", x"89", x"0d", x"bf", x"e6", x"42", x"68", x"41", x"99", x"2d", x"0f", x"b0", x"54", x"bb", x"16");

begin

subRotW(8*(i+1)-1 downto 8*i) <= subKey(conv_integer(rotW(8*(i+1)-1 downto 8*i)));

end generate;

temp <=	subRotW xor (Rcon(conv_integer(roundCounter(5 downto 2))) & x"000000") when roundCounter(1 downto 0) = "00" else

w(3);

--end keyExpansionPipe asynchron circuitry

expansionKey <= w(0) & w(1) & w(2) & w(3);

--------------------

--ENCRYPTION--------

--------------------

--ADD BYTES TRANSFORM

add <=	plain xor expansionKey when roundCounter = "000000" else

mix xor expansionKey;

--SUB BYTES TRANSFORM

--encryption's synchron circuitry

subBytes: for i in 0 to 15 generate

type subT is array(0 to 255) of std_logic_vector(7 downto 0);

constant subRam : subT := (

x"63", x"7c", x"77", x"7b", x"f2", x"6b", x"6f", x"c5", x"30", x"01", x"67", x"2b", x"fe", x"d7", x"ab", x"76",

x"ca", x"82", x"c9", x"7d", x"fa", x"59", x"47", x"f0", x"ad", x"d4", x"a2", x"af", x"9c", x"a4", x"72", x"c0",

x"b7", x"fd", x"93", x"26", x"36", x"3f", x"f7", x"cc", x"34", x"a5", x"e5", x"f1", x"71", x"d8", x"31", x"15",

x"04", x"c7", x"23", x"c3", x"18", x"96", x"05", x"9a", x"07", x"12", x"80", x"e2", x"eb", x"27", x"b2", x"75",

x"09", x"83", x"2c", x"1a", x"1b", x"6e", x"5a", x"a0", x"52", x"3b", x"d6", x"b3", x"29", x"e3", x"2f", x"84",

x"53", x"d1", x"00", x"ed", x"20", x"fc", x"b1", x"5b", x"6a", x"cb", x"be", x"39", x"4a", x"4c", x"58", x"cf",

x"d0", x"ef", x"aa", x"fb", x"43", x"4d", x"33", x"85", x"45", x"f9", x"02", x"7f", x"50", x"3c", x"9f", x"a8",

x"51", x"a3", x"40", x"8f", x"92", x"9d", x"38", x"f5", x"bc", x"b6", x"da", x"21", x"10", x"ff", x"f3", x"d2",

x"cd", x"0c", x"13", x"ec", x"5f", x"97", x"44", x"17", x"c4", x"a7", x"7e", x"3d", x"64", x"5d", x"19", x"73",

x"60", x"81", x"4f", x"dc", x"22", x"2a", x"90", x"88", x"46", x"ee", x"b8", x"14", x"de", x"5e", x"0b", x"db",

x"e0", x"32", x"3a", x"0a", x"49", x"06", x"24", x"5c", x"c2", x"d3", x"ac", x"62", x"91", x"95", x"e4", x"79",

x"e7", x"c8", x"37", x"6d", x"8d", x"d5", x"4e", x"a9", x"6c", x"56", x"f4", x"ea", x"65", x"7a", x"ae", x"08",

x"ba", x"78", x"25", x"2e", x"1c", x"a6", x"b4", x"c6", x"e8", x"dd", x"74", x"1f", x"4b", x"bd", x"8b", x"8a",

x"70", x"3e", x"b5", x"66", x"48", x"03", x"f6", x"0e", x"61", x"35", x"57", x"b9", x"86", x"c1", x"1d", x"9e",

x"e1", x"f8", x"98", x"11", x"69", x"d9", x"8e", x"94", x"9b", x"1e", x"87", x"e9", x"ce", x"55", x"28", x"df",

x"8c", x"a1", x"89", x"0d", x"bf", x"e6", x"42", x"68", x"41", x"99", x"2d", x"0f", x"b0", x"54", x"bb", x"16");

begin

process(clock)

begin

if rising_edge(clock) then

if roundCounter(1 downto 0) = "00" and roundCounter < (x"a" & "00") then

sub(8*(i+1)-1 downto 8*i) <= subRam(conv_integer(add(8*(i+1)-1 downto 8*i)));

end if;

end if;

end process;

end generate;

--SHIFT ROWS TRANSFORM

shiftGenCol:for row in 0 to 3 generate

begin

shiftGenRow:for col in 0 to 3 generate

begin

--shifter input signals

shiftInState(3-col)(3-row) <= sub(32*col+8*(row+1)-1 downto 32*col+8*row);

--shifter output signals

shift(32*col+8*(row+1)-1 downto 32*col+8*row) <= shiftOutState(3-col)(3-row);

end generate;

end generate;

shiftOutState(0)(0) <= shiftInState(0)(0);

shiftOutState(0)(1) <= shiftInState(1)(1);

shiftOutState(0)(2) <= shiftInState(2)(2);

shiftOutState(0)(3) <= shiftInState(3)(3);

shiftOutState(1)(0) <= shiftInState(1)(0);

shiftOutState(1)(1) <= shiftInState(2)(1);

shiftOutState(1)(2) <= shiftInState(3)(2);

shiftOutState(1)(3) <= shiftInState(0)(3);

shiftOutState(2)(0) <= shiftInState(2)(0);

shiftOutState(2)(1) <= shiftInState(3)(1);

shiftOutState(2)(2) <= shiftInState(0)(2);

shiftOutState(2)(3) <= shiftInState(1)(3);

shiftOutState(3)(0) <= shiftInState(3)(0);

shiftOutState(3)(1) <= shiftInState(0)(1);

shiftOutState(3)(2) <= shiftInState(1)(2);

shiftOutState(3)(3) <= shiftInState(2)(3);

--MIX COLUMNS TRANSFORM

mixColumns : for i in 0 to 3 generate

signal xi, xo : std_logic_vector(31 downto 0);

signal s0i, s1i, s2i, s3i, s0o, s1o, s2o, s3o : std_logic_vector(7 downto 0);

signal s0x2, s1x2, s2x2, s3x2, s0x3, s1x3, s2x3, s3x3 : std_logic_vector(7 downto 0);

begin

xi <= shift(32*(i+1)-1 downto 32*i);

s0i <= xi(31 downto 24);

s1i <= xi(23 downto 16);

s2i <= xi(15 downto 8);

s3i <= xi(7 downto 0);

--multiplication by two over Galois field

s0x2 <= s0i(6 downto 0) & '0' when s0i(7) = '0' else (s0i(6 downto 0) & '0') xor "00011011";

s1x2 <= s1i(6 downto 0) & '0' when s1i(7) = '0' else (s1i(6 downto 0) & '0') xor "00011011";

s2x2 <= s2i(6 downto 0) & '0' when s2i(7) = '0' else (s2i(6 downto 0) & '0') xor "00011011";

s3x2 <= s3i(6 downto 0) & '0' when s3i(7) = '0' else (s3i(6 downto 0) & '0') xor "00011011";

--multiplication by three over Galois field: addition of times 1 and times 2

s0x3 <= s0i xor s0x2;

s1x3 <= s1i xor s1x2;

s2x3 <= s2i xor s2x2;

s3x3 <= s3i xor s3x2;

--addition over Galois field

s0o <= s0x2 xor s1x3 xor s2i xor s3i;

s1o <= s0i xor s1x2 xor s2x3 xor s3i;

s2o <= s0i xor s1i xor s2x2 xor s3x3;

s3o <= s0x3 xor s1i xor s2i xor s3x2;

xo <= s0o & s1o & s2o & s3o;

mixOut(32*(i+1)-1 downto 32*i) <= xo;

end generate;

mix <=	shift when roundCounter(5 downto 2) = x"a" else

mixOut;

--------------------

--OUTPUT------------

--------------------

cipher <= add;

end Behavioral;