--AES core testbench

library ieee;

use ieee.std_logic_1164.all;

entity aes128keyTB is

end aes128keyTB;

architecture behavior of aes128keyTB is

component aes128key

port(

reset  : in  std_logic;

clock  : in  std_logic;

empty  : out std_logic;

load   : in  std_logic;

key    : in  std_logic_vector(127 downto 0);

plain  : in  std_logic_vector(127 downto 0);

ready  : out std_logic;

cipher : out std_logic_vector(127 downto 0)

);

end component;

signal reset  : std_logic := '0';

signal clock  : std_logic := '0';

signal load   : std_logic := '0';

signal key    : std_logic_vector(127 downto 0) := (others => '0');

signal plain  : std_logic_vector(127 downto 0) := (others => '0');

signal empty  : std_logic;

signal ready  : std_logic;

signal cipher : std_logic_vector(127 downto 0);

constant clock_period : time := 10 ns;

begin

uut : aes128key

port map (

reset  => reset,

clock  => clock,

empty  => empty,

load   => load,

key    => key,

plain  => plain,

ready  => ready,

cipher => cipher

);

-- clock

clock_process : process

begin

clock <= '0';

wait for clock_period/2;

clock <= '1';

wait for clock_period/2;

end process;

-- stimulus

stim_proc : process

begin

reset <= '1';

wait for 100 ns;

reset <= '0';

wait for clock_period*10;

-- FIPS-197 AES-128 test vector

key   <= x"000102030405060708090a0b0c0d0e0f";

plain <= x"00112233445566778899aabbccddeeff";

load  <= '1';

wait for clock_period;

load  <= '0';

-- wait long enough for all rounds in this core

wait for clock_period*80;

report "Simulation finished, cipher = " severity note;

wait;

end process;

end behavior;