#include <stdio.h>

#include "xparameters.h"

#include "xil_printf.h"

#include "xil_io.h"

#include "xtime_l.h"

#include "aes.h"

#define AES_BASEADDR   XPAR_MYIP_12_0_S00_AXI_BASEADDR

#define AES_REG_P0     0x00  // [127:96]

#define AES_REG_P1     0x04  // [95:64]

#define AES_REG_P2     0x08  // [63:32]

#define AES_REG_P3     0x0C  // [31:0]

#define AES_REG_CTRL   0x10  // bit0: start/ready

#define AES_WRITE(off, data)  Xil_Out32(AES_BASEADDR + (off), (data))

#define AES_READ(off)         Xil_In32 (AES_BASEADDR + (off))

//#define DEBUG_HW_ACCEL

static const uint8_t aes_key[16] = {

0x00,0x01,0x02,0x03,

0x04,0x05,0x06,0x07,

0x08,0x09,0x0a,0x0b,

0x0c,0x0d,0x0e,0x0f

};

static const uint8_t aes_plaintext[16] = {

0x00,0x11,0x22,0x33,

0x44,0x55,0x66,0x77,

0x88,0x99,0xaa,0xbb,

0xcc,0xdd,0xee,0xff

};

// ---------- HW AES one block ----------

void aes_hw_encrypt_block(const uint8_t *pt, uint8_t *ct)

{

u32 p0 = ((u32)pt[0]  << 24) | ((u32)pt[1] << 16) | ((u32)pt[2] << 8) | pt[3];

u32 p1 = ((u32)pt[4]  << 24) | ((u32)pt[5] << 16) | ((u32)pt[6] << 8) | pt[7];

u32 p2 = ((u32)pt[8]  << 24) | ((u32)pt[9] << 16) | ((u32)pt[10] << 8) | pt[11];

u32 p3 = ((u32)pt[12] << 24) | ((u32)pt[13] << 16) | ((u32)pt[14] << 8) | pt[15];

AES_WRITE(AES_REG_P0, p0);

AES_WRITE(AES_REG_P1, p1);

AES_WRITE(AES_REG_P2, p2);

AES_WRITE(AES_REG_P3, p3);

AES_WRITE(AES_REG_CTRL, 0x00000001);

while ((AES_READ(AES_REG_CTRL) & 0x1) == 0) {

;

}

u32 c1 = AES_READ(AES_REG_P1);

u32 c2 = AES_READ(AES_REG_P2);

u32 c3 = AES_READ(AES_REG_P3);

u32 c0 = AES_READ(AES_REG_P0);

//#ifdef DEBUG_HW_ACCEL

// xil_printf("HW words: %08lx %08lx %08lx %08lx\r\n",

//(unsigned long)c0, (unsigned long)c1,

// (unsigned long)c2, (unsigned long)c3);

//#endif

ct[0]  = (uint8_t)(c0 >> 24);

ct[1]  = (uint8_t)(c0 >> 16);

ct[2]  = (uint8_t)(c0 >>  8);

ct[3]  = (uint8_t)(c0      );

ct[4]  = (uint8_t)(c1 >> 24);

ct[5]  = (uint8_t)(c1 >> 16);

ct[6]  = (uint8_t)(c1 >>  8);

ct[7]  = (uint8_t)(c1      );

ct[8]  = (uint8_t)(c2 >> 24);

ct[9]  = (uint8_t)(c2 >> 16);

ct[10] = (uint8_t)(c2 >>  8);

ct[11] = (uint8_t)(c2      );

ct[12] = (uint8_t)(c3 >> 24);

ct[13] = (uint8_t)(c3 >> 16);

ct[14] = (uint8_t)(c3 >>  8);

ct[15] = (uint8_t)(c3      );

}

// ---------- SW AES one block ----------

void aes_sw_encrypt_block(const uint8_t *pt, uint8_t *ct)

{

struct AES_ctx ctx;

uint8_t buf[16];

for (int i = 0; i < 16; i++)

buf[i] = pt[i];

AES_init_ctx(&ctx, aes_key);

AES_ECB_encrypt(&ctx, buf);

for (int i = 0; i < 16; i++)

ct[i] = buf[i];

}

// ---------- helpers ----------

void print_block(const char *label, const uint8_t *b)

{

xil_printf("%s", label);

for (int i = 0; i < 16; i++)

xil_printf("%02x", b[i]);

xil_printf("\r\n");

}

// ---------- main ----------

int main(void)

{

XTime t1, t2;

uint8_t ct_hw[16];

uint8_t ct_sw[16];

unsigned long long hw_ticks, sw_ticks;

double hw_us, sw_us;

xil_printf("AES HW/SW Timing Test\r\n");

// HW AES

XTime_GetTime(&t1);

aes_hw_encrypt_block(aes_plaintext, ct_hw);

XTime_GetTime(&t2);

hw_ticks = (unsigned long long)(t2 - t1);

hw_us = (double)hw_ticks / (COUNTS_PER_SECOND / 1000000.0);

// SW AES

XTime_GetTime(&t1);

aes_sw_encrypt_block(aes_plaintext, ct_sw);

XTime_GetTime(&t2);

sw_ticks = (unsigned long long)(t2 - t1);

sw_us = (double)sw_ticks / (COUNTS_PER_SECOND / 1000000.0);

// Pack SW bytes into words for fair compare

u32 sw0 = ((u32)ct_sw[0]  << 24) | ((u32)ct_sw[1] << 16) | ((u32)ct_sw[2] << 8) | ct_sw[3];

u32 sw1 = ((u32)ct_sw[4]  << 24) | ((u32)ct_sw[5] << 16) | ((u32)ct_sw[6] << 8) | ct_sw[7];

u32 sw2 = ((u32)ct_sw[8]  << 24) | ((u32)ct_sw[9] << 16) | ((u32)ct_sw[10] << 8) | ct_sw[11];

u32 sw3 = ((u32)ct_sw[12] << 24) | ((u32)ct_sw[13] << 16) | ((u32)ct_sw[14] << 8) | ct_sw[15];

// Read HW words again for comparison

u32 c0 = AES_READ(AES_REG_P0);

u32 c1 = AES_READ(AES_REG_P1);

u32 c2 = AES_READ(AES_REG_P2);

u32 c3 = AES_READ(AES_REG_P3);

int match_words = (c0 == sw0) && (c1 == sw1) && (c2 == sw2) && (c3 == sw3);

print_block("Plaintext : ", aes_plaintext);

print_block("HW cipher : ", ct_hw);

print_block("SW cipher : ", ct_sw);

xil_printf("HW words: %08lx %08lx %08lx %08lx\r\n",

(unsigned long)c0, (unsigned long)c1,

(unsigned long)c2, (unsigned long)c3);

xil_printf("SW words: %08lx %08lx %08lx %08lx\r\n",

(unsigned long)sw0, (unsigned long)sw1,

(unsigned long)sw2, (unsigned long)sw3);

xil_printf("MATCH(words): %s\r\n", match_words ? "YES" : "NO");

printf("HW: %llu ticks, %.3f us\r\n", hw_ticks, hw_us);

printf("SW: %llu ticks, %.3f us\r\n", sw_ticks, sw_us);

if (hw_us > 0.0)

printf("Speedup (SW/HW): %.2fx\r\n", sw_us / hw_us);

while (1) ;

return 0;

}